| Reg. No. : Question Paper Code: 47012 |
| --- |
| B.E/B.TECH DEGREE END SEMESTER EXAMINATIONS , Nov 2017 |  |  |  |  |  |  |  |  |  |
| Seventh Semester |  |  |  |  |  |  |  |  |  |
| Civil Engineering |  |  |  |  |  |  |  |  |  |
| 14UCE702 – ADVANCED STRUCTURAL DESIGN (Regulation 2014) (Note: Use of IS 456:2000, SP 16:1980, IS 3370(Part –II):2009, IS 3370(Part –IV):1967, IS 800:2007, SP 6-1:1964 and IRC 21:2000 are permitted in the End Semester Examinations) |  |  |  |  |  |  |  |  |  |
| Duration: 3 Hours |  |  |  |  | Maximum: 100 Marks |  |  |  |  |
| PART -- A (10 X 1 = 10 Marks) (Answer all Questions) |  |  |  |  |  |  |  |  |  |
| 1. |  |  | High strength bolts are designed on the basis of |  |  |  |  |  |  |
|  |  |  | (a) Friction |  |  | (b) Tension |  |  |  |
|  |  |  | (c) Compression |  |  | (d) Shear |  |  |  |
| 2. |  |  | Permissible shear for a field weld is reduced by |  |  |  |  |  |  |
|  |  |  | (a) 5% |  |  |  | (b) 80% |  |  |
|  |  |  | (c) 15% |  |  |  | (d) 20% |  |  |
| 3. |  |  | The under direct tension members are designed by |  |  |  |  |  |  |
|  |  |  | (a) Dr.Reissners Method |  |  |  | (b) Carpenter's Method |  |  |
|  |  |  | (c) I.S Code method |  |  |  | (d) Both (a) & (b) |  |  |
| 4. |  |  | The percentage of reinforcement in base or floor slab resisting directly on ground should not be less than |  |  |  |  |  |  |
|  |  |  | (a) 0.15% bD |  |  |  | (b) 0.18% bD |  |  |
|  |  |  | (c) 0.20% bD |  |  |  | (d) 0.13% bD |  |  |
| 5. |  |  | Standard three sided culverts range from |  |  |  |  |  |  |
|  |  |  | (a) 8 to 48 m length |  |  |  | (b) 6 to 36 m length |  |  |
|  |  |  | (c) 12 to 60m length |  |  |  | (d) 5 to 30m length |  |  |
| 6. |  |  | For liquid faces of parts of members either in contact with the liquid (such as inner faces or roof slab) the minimum cover to all reinforcement should be |  |  |  |  |  |  |
|  |  |  | (a) 25mm |  |  | (b) 15mm |  |  |  |
|  |  |  | (c) 40mm |  |  | (d) 45mm |  |  |  |
| 7. |  |  | Maximum factor of safety against sliding of the retaining wall should not be more than |  |  |  |  |  |  |
|  |  |  | (a) 1.5 |  |  | (b) 2 |  |  |  |
|  |  |  | (c) 3 |  |  | (d) 4 |  |  |  |
| 8. |  |  | Main reinforcement in the heel of cantilever retaining wall is placed |  |  |  |  |  |  |
|  |  |  | (a) Near the lower face |  |  |  | (b) Near to upper face |  |  |
|  |  |  | (c) At the center of the cross – section |  |  |  | (d) At 2/3rd depth of heel |  |  |
| 9. |  |  | Allowable tension in steel under wind load condition that can exceed that under live load conditions is |  |  |  |  |  |  |
|  |  |  | (a) 25% |  |  |  |  | (b) 33% |  |
|  |  |  | (c) 50% |  |  |  |  | (d) None of the above |  |
| 10. |  |  | Which of the following is not a compression member? |  |  |  |  |  |  |
|  |  |  | (a) Strut |  |  |  |  | (b) Tie |  |
|  |  |  | (c) Rafter |  |  |  |  | (d) Boom |  |
| PART - B (5 X 2= 10 Marks) |  |  |  |  |  |  |  |  |  |
| 11. |  |  | What are the advantages of welded joints? |  |  |  |  |  |  |
| 12. |  |  | State the necessity or providing bracing in a staging of water tank? |  |  |  |  |  |  |
| 13. |  |  | Write the short notes on IRC Loadings. |  |  |  |  |  |  |
| 14. |  |  | State the necessity of providing shear key in retaining wall. |  |  |  |  |  |  |
| 15. |  |  | Enumerate the principle of design of a purlin. |  |  |  |  |  |  |
| PART -C ( 5 X 16 = 80 Marks) |  |  |  |  |  |  |  |  |  |
| 16 |  | (a) |  | A tie member of a roof truss consists of two ISA 10075, 8mm. The angles are connected to either side of a 10 mm gusset plates and the member is subjected to a working pull of 300 kN. Design the welded connection. Assume the connections are made in workshop. |  |  |  |  | (16) |
| Or |  |  |  |  |  |  |  |  |  |
|  | (b) |  |  | A beam ISMB 500 @ 86.9 kg/m transmits an end reaction of 250 kN to the web of a column ISHB 300 @ 63.0 kg/m. Design a stiffened welded seat connection using field weld. |  |  |  |  | (16) |
| 17 | (a) |  |  | Design an elevated cylindrical steel tank with hemispherical bottom for 1,60,000 liters capacity. The tank has conical roof. The ring beam of the tank is at a height of 10 m from the ground level. The tank is to be built at Chennai. Take fy = 250 N/mm2 |  |  |  |  | (16) |
| Or |  |  |  |  |  |  |  |  |  |
|  | (b) |  |  | Design a circular tank with a flexible base for capacity of 500000 liters. The depth of water is to be 4m.Free board=200mm.use M20 concrete and grade Fe415 grade steel. Permissible direct tensile stress in concrete =1.2N/mm2.Permissible stress in steel in direct tension=100N/mm2.Sketch the details of reinforcements in tank walls. |  |  |  |  | (16) |
| 18 | (a) |  |  | Design a solid slab bridge required for a highway road having the following data. Width of carriage way=7.5m Clear span = 6m Loading = IRC Class AA Width of kerb= 600mm Width of bearings= 500mm Materials M30 grade of concrete and Fe415 steel. |  |  |  |  | (16) |
| Or |  |  |  |  |  |  |  |  |  |
|  | (b) |  |  | Design a RCC Box Culvert having a clear way of vent size 3.5m x 3.5m Live load and dead load on the culvert is 30kN/m2 and 10kN/m2 respectively. Unit weight of soil is 16kN/m3. Angle of repose is 30. Use M20 grade of concrete and Fe415 grade steel. Assume any other data if necessary. |  |  |  |  | (16) |
| 19 | (a) |  |  | Design the vertical stem of a counter - fort retaining wall if the heights of the wall above the ground level are 5.60 m. The safe bearing capacity of the soil is 175 kN/m2. The unit weight of soil is 18 kN/m3. The angle of repose of the soil is 30 degree. The co – efficient of friction between the soil and concrete is 0.50. Assume the spacing of the counter – fort as 3m. Adopt M30 concrete and Fe 415 steel. |  |  |  |  | (16) |
| Or |  |  |  |  |  |  |  |  |  |
|  | (b) |  |  | Design a cantilever retaining wall to retain earth embankment 4.5 m above the ground level. The density of earth is 18 kN/m3 and its angle of repose of is 30ᵒ. The embankment is horizontal at its top. The safe bearing capacity may be taken as 200 kN/m2 and the co efficient of friction between soils and concrete is 0.5. Use M20 grade concrete and Fe 415 grade steel reinforcement. |  |  |  |  | (16) |
| 20 | (a) |  |  | Determine the basic wind intensity for an industrial building situated in Chennai using the following data: Life of the structure = 50 years Terrain category = 2 Size of the building = 20 m x 40 m Height of the eave board = 10 m Topography = slpoe < 3º Slope = 1 in 4. |  |  |  |  | (16) |
| Or |  |  |  |  |  |  |  |  |  |
|  | (b) |  |  | Design the purlin for the following specification: Span of truss = 12 m c/c Pitch = 1/5 of span Spacing of truss = 5 m c/c Spacing of purlin = 1.5 m c/c Load from roofing materials etc., = 200 N/mm2. Wind load = 1200 N/m2. Use angle section. |  |  |  |  | (16) |